OPTIMZATION OF TAPROOT FIBRE MANUFACTURING USING

AVAILABLE RESOURCES

Prepared For: Dr. Scott Flemming

Prepared by:

JAFIR SULAIMAN SATHATH ALI (B00989993)

Table of Contents

Introduction	2

Background	2

Workbench	2

Breaker:	3

Scutching  Machine	3

Button	4

PROBLEM STATEMENT	7

Our Model	8

ONE WORKER AND 2 CLIPS(JOBS)	9

ONE worker and 3 clips	10

ONE WORKER AND 4 CLIPS	11

ONE WORKER AND FIVE CLIPS(JOBS)	12

ONE WORKER AND SIX CLIPS(JOBS)	13

Finding Optimum clips(jobs) with available resource.	14

Improvement (without increasing available resources)	15

ONE WORKER AND 2 CLIPS(JOBS)	17

ONE WORKER AND 3 CLIPS(JOBS)	17

ONE WORKER AND 4 CLIPS(JOBS)	18

ONE WORKER AND 5 CLIPS(JOBS)	18

ONE WORKER AND 6 CLIPS(JOBS)	19

Finding Optimum clips(jobs) with available resource after improvements	20

Future study and suggestion	21

Conclusion	22

Acknowledgement	22

## Introduction

The main focus on this project is to improve the scutching process at TapRoot farms. Before explaining the method used we will describe the production process in more detail.

## Background

Flax is a natural agricultural product. It has to undergo sequence of many tasks to get a final output of long-line linen fibre.  Long-line can be converted to make various textiles. It is a an industry that is growing steadily due to increasing demand for natural fibres which are eco-friendly.

Workbench

It is a initial stage of process where worker rests the plant fibre on the table.Here ,worker attaches the fibre in a clip and screw it up firmly for process.Clip which helps to hold fibre very effectively.Second time,we have to uncrew clip in this workbench.After that, we have to flip it other side of fibre and start screw it in that clip.

## Breaker:

In that factory ,there are three machines.Breaker is one of the machine. It is located near to workbench and Scutching machine.It helps to separate woody layer from fibre.Make it even better for futher process.Literally,it breaks hard layer of the root.

Figure 1: Breaker machine at TapRoot Farms

## Scutching  Machine

It removes shives after breaking and  helps to clean  fibre for hackling process.

Scutching Machine.

Button

Clip automatically moves from breaker to scutcher after a button is pressed to go to the 2nd of 3 steps.

.

Flax is fixed in the clip which helps to carry the flax firmly all through the process.Clip has to go two through twice because it has to be flipped upside down after the first time.

Fibre is attached to the clip

Hackler

It Combs and align fibres and separates long fibre from short fibre.

Table 1: Machine and Task processing times

Note: P1(A),P7(A) ,P2(B) and P7(B)  which are workbench process time , split into two processing time for priotiozing machine works and other short processing time in other clips

| Tasks | Sequence | Processing time(minutes) |
| --- | --- | --- |
| Workbench | P1(A) | 2.2 |
| Workbench | P1(B) | 0 |
| Breaker | P2 | 1.34 |
| Button | P3 | 0.17 |
| Sctuching machine | P4 | 0.77 |
| Moving output to hackler | P5 | 1.2 |
| hackler | P6 | 2 |
| Workbench | P7(A) | 2.25 |
| Workbench | P7(B) | 0 |
| breaker | P8 | 1.34 |
| Button | P9 | 0.17 |
| Scutching machine | P10 | 0.77 |
| Moving output to hackler | P11 | 1.2 |
| hackler | P12 | 2 |
| workbench | P13 | 2.25 |

## PROBLEM STATEMENT

For this project, we wished to find out what number of clips would result in the greatest production in terms of grams of long-line linen produced.  In this project we assumed the following:

- One breaker

- One sctuching machine

- One work bench

- Single worker

In order to find out how the number of clips affected the total output we used an LP that calculated the makespan when a given number of clips was used.  We used a video of the process to estimate the processing times for each part of the process.

.

## Our Model

Objectives :

- Min Cmax

Constraints:

Precedence Constraints:

- SI+1 > =CI

- SJ>=CI+1

Avoiding Overlap human work in one clip with other clip

- CJ – SI+1- BIG M NUMBER*(1- PJ)<=0

- CI+1 -SJ - BIG M NUMBER*(1- PI)<=0

(Likewise we have to do it for all human process)

- P1(A)+P1(B)=2.2

- P7(A)+P7(B)=2.25

- PI+PJ=1

- Human completion time= starting time of machine

- Decision Variable are all starting time (S) of process and human processing time for workbench(P1(A),P1(B),P7(A) and P7(B) )which are divided into two parts to stop and pause after prioritize other short processing time.

- C denotes completion time.

- P denotes processing time.

- P1,P3,P5,P7,P9,P11,P13 – This process is done by worker

- P2,P4,P6,P10,P12- This processes are machine.

Using this model, we get gantt chart.

## ONE WORKER AND 2 CLIPS(JOBS)

CMAX:20.7

## ONE worker and 3 clips

CMAX:29.86

## ONE WORKER AND 4 CLIPS

CMAX:29.86

## ONE WORKER AND FIVE CLIPS(JOBS)

CMAX:47.2

## ONE WORKER AND SIX CLIPS(JOBS)

CMAX:56.63

| clips x and worker(one) | Cmax |
| --- | --- |
| 1 | 17.66 |
| 2 | 20.7 |
| 3 | 29.86 |
| 4 | 37.76 |
| 5 | 47.2 |
| 6 | 56.63 |

## Finding Optimum clips(jobs) with available resource.

| clips x and worker(one) | per clip hold 500 kg | cmax | batches( 8hours per day) |
| --- | --- | --- | --- |
| 1 | 250 | 17.66 | 27.18006795 |
| 2 | 500 | 20.7 | 23.1884058 |
| 3 | 750 | 29.86 | 16.07501674 |
| 4 | 1000 | 37.76 | 12.71186441 |
| 5 | 1250 | 47.2 | 10.16949153 |
| 6 | 1500 | 56.63 | 8.476072753 |

| per hour salary for worker(18cad) | batch | output 10 percent of input | actual output |
| --- | --- | --- | --- |
| 144 | 27 | 25 | 675 |
| 144 | 23 | 100 | 2300 |
| 144 | 16 | 225 | 3600 |
| 144 | 12 | 400 | 4800 |
| 144 | 10 | 625 | 6250 |
| 144 | 8 | 900 | 7200 |

Selling price: cad 12

Purchasing cost: cad 6

| clips | purchasing cost of input | Revenue | Profit | Profit per unit |
| --- | --- | --- | --- | --- |
| 1 | 1.5 | 8.1 | 6.6 | 4.4 |
| 2 | 3 | 27.6 | 24.6 | 8.2 |
| 3 | 4.5 | 43.2 | 38.7 | 8.6 |
| 4 | 6 | 57.6 | 51.6 | 8.6 |
| 5 | 7.5 | 75 | 67.5 | 9 |
| 6 | 9 | 86.4 | 77.4 | 8.6 |

Based on profit per unit,5th clips is optimum.

## Improvement (without increasing available resources)

Now we bring the work bech to centre .so P1 and P7 process time are 2.3 minutes and 2.2 minutes respectively.Using Button to move output one place to other with help of implement conveyor in between machines from sctuching machine  to Hackler.Now processing time time for P3,P5,P9,P11 – 0.12 minutes

| Tasks | Sequence | Processing time(minutes) |
| --- | --- | --- |
| Workbench | P1(A) | 0 |
| Workbench | P1(B) | 2.23 |
| Breaker | P2 | 1.34 |
| Button | P3 | 0.12 |
| Sctuching machine | P4 | 0.77 |
| Moving output to hackler | P5 | 0.12 |
| hackler | P6 | 2 |
| Workbench | P7(A) | 0.2 |
| Workbench | P7(B) | 2 |
| breaker | P8 | 1.34 |
| Button | P9 | 0.12 |
| Scutching machine | P10 | 0.77 |
| Moving output to hackler | P11 | 0.12 |
| hackler | P12 | 2 |
| Workbench | P13 | 2.25 |

Our Gantt looks after improvements

## ONE WORKER AND 2 CLIPS(JOBS)

CMAX:18.19

## ONE WORKER AND 3 CLIPS(JOBS)

CMAX:24.09

## ONE WORKER AND 4 CLIPS(JOBS)

CMAX:30.09

## ONE WORKER AND 5 CLIPS(JOBS)

CMAX:37.02

## ONE WORKER AND 6 CLIPS(JOBS)

CMAX:44.83

| clips x and worker(one) | cmax |
| --- | --- |
| 1 | 15.38 |
| 2 | 18.19 |
| 3 | 24.09 |
| 4 | 30.09 |
| 5 | 37.02 |
| 6 | 44.83 |

## Finding Optimum clips(jobs) with available resource after improvements

| clips x and worker(one) | per clip hold 250 kg | cmax | batches( 8hours per day) | per hour salary for worker(18cad) |
| --- | --- | --- | --- | --- |
| 1 | 250 | 15.38 | 31.20936281 | 144 |
| 2 | 500 | 18.19 | 26.38812534 | 144 |
| 3 | 750 | 24.09 | 19.9252802 | 144 |
| 4 | 1000 | 30.09 | 15.95214357 | 144 |
| 5 | 1250 | 37.02 | 12.96596434 | 144 |
| 6 | 1500 | 44.83 | 10.70711577 | 144 |

| batch round | output=10percent of actual input | output |
| --- | --- | --- |
| 31 | 25 | 775 |
| 26 | 100 | 2600 |
| 19 | 225 | 4275 |
| 15 | 400 | 6000 |
| 12 | 625 | 7500 |
| 10 | 900 | 9000 |

Selling price: cad 12

Purchasing cost: cad 6

| clips | purchasing cost of input | Revenue | Profit | Profit per unit |
| --- | --- | --- | --- | --- |
| 1 | 1.5 | 9.3 | 7.8 | 5.2 |
| 2 | 3 | 31.2 | 28.2 | 9.4 |
| 3 | 4.5 | 51.3 | 46.8 | 10.4 |
| 4 | 6 | 72 | 66 | 11 |
| 5 | 7.5 | 90 | 82.5 | 11 |
| 6 | 9 | 108 | 99 | 11 |

Based on profit per unit, 4 ,5 and 6 clips are optimum.

## Future study and suggestion

In this project,  workbench process divided into two parts to act like stop and resume in order to prioritise other jobs.In future, divide this workbench process more than 2 parts.Morover, try to increase the resource and analyse which clips are optimum.We can clearly see that after doing improvement.cmax for every clips get reduced .Trying to implement in real that help to optimize this process.

# Conclusion

Using our model with available resource without doing any improvements.Doing 5 clips(jobs) is optimum.

## Acknowledgement

I got help from Ai for generating python code for displaying gantt chart.